Date:

ID:

Name:

Surname:

Signature:

HLK201 HELICOPTER DESIGN

QUESTIONS SET 1

FUNDAMENTALS OF HELICOPTER DESIGN

Q1. (7p) Which component generates lift in a helicopter?

a) Fuselage
b) Tail rotor
c) Main rotor
d) Engine

Q2. (8p) Which cockpit control primarily controls yaw?

a) Cyclic
b) Collective
c) Pedals
d) Throttle

Q3. (8p) A helicopter hovers with thrust T=10 000 N and rotor disk area A=50 m2.
What is the disk loading?

DL=TA

a) 100 N/m2
b) 200  N/m2
c) 500  N/m2
d) 5 000  N/m2

Q4. (15p) In hover, the velocity distribution over the rotor disk is:

a) Asymmetric
b) Zero everywhere
c) Symmetric
d) Random

Q5. (8p) What flight regime allows a helicopter to remain stationary relative to the ground?

a) Forward flight
b) Autorotation
c) Hover
d) Axial descent

Q6. (8p) Which power component exists whenever the rotor rotates, regardless of forward speed?

a) Induced power
b) Profile power
c) Parasite power
d) Excess power

Q7. (15p) Which statement is true?

a) Translational lift exists in hover
b) Hover requires less power than forward flight
c) Hover requires maximum induced power
d) Parasite power dominates in hover

Q8. (8p) Standard air density at sea level is approximately:

a) 1.0  kg/m3
b) 1.18 kg/m3
c) 1.225 kg/m3
d) 1.30 kg/m3

Q9. (8p) Which rotor configuration does NOT require a tail rotor?

a) Single rotor
b) Tandem rotor
c) Conventional helicopter
d) Single-rotor UAV

Q10. (15p) The collective control primarily:

a) Changes rotor RPM
b) Changes blade angle of attack equally
c) Controls yaw
d) Tilts the rotor disk

Q11. (+ 20p - Bonus Question)

A helicopter hovers at sea level with:

- Thrust T = 40 000 N

- Rotor radius R = 6 m  A = πR2

- Air density ρ=1.225 kg/m3

Using momentum theory:

ϑi=T2ρA,            Pi=Tϑi

What are ϑi and Pi (approximately)?

a) ϑi≈6.9 m/s,    Pi≈276 kW

b) ϑi≈8.5 m/s,    Pi≈340 kW

c) ϑi≈10.2 m/s,    Pi≈408 kW

d) ϑi≈12.8 m/s,    Pi≈512 kW