Date:

ID:

Name:

Surname:

Signature:

HLK201 HELICOPTER DESIGN

QUESTIONS SET 2

ROTORCRAFT PERFORMANCE AND CONTROL

### Q1. (7 p) Why does a helicopter require maximum power in hover?

a) Parasite drag is maximum
b) Translational lift is absent
c) Profile power is zero
d) Engine efficiency is lowest

### Q2. (8 p) In forward flight, which blade experiences higher relative velocity?

a) Retreating blade
b) Advancing blade
c) Root section
d) Hub region

### Q3. (15 p) As altitude increases, a helicopter will first lose its ability to:

a) Fly forward
b) Hover
c) Autorotate
d) Produce lift

### Q4. (8 p) Induced power in hover is: Pi=Tϑi

If T=15 000 N and ϑi=6 m/s, what is Pi?

a) 45 kW
b) 60 kW
c) 90 kW
d) 120 kW

### Q5. (8 p)

Which aerodynamic theory treats the rotor as an actuator disk?

a) Blade Element Theory
b) Momentum Theory
c) CFD
d) Lifting Line Theory

### Q6. (15 p)

Which quantity cannot be obtained from Momentum Theory alone?

a) Induced velocity
b) Induced power
c) Thrust
d) Blade twist distribution

### Q7. (8 p)

Disk loading is defined as:

a) Power per unit area
b) Thrust per unit area
c) Torque per unit radius
d) Lift per blade

### Q8. (8 p)

Which control input mitigates overturning moment in forward flight?

a) Pedals
b) Throttle
c) Cyclic
d) Collective

### Q9. (15 p)

Which power component mainly depends on rotor blade drag?

a) Induced power
b) Profile power
c) Parasite power
d) Excess power

### Q10. (8 p)

Why are rotor blades thicker near the root?

a) To reduce induced power
b) To reduce tip Mach number
c) To carry higher structural loads
d) To reduce parasite drag

### Q11. (+20 p – Bonus Question)

At sea level, a helicopter has:

- Available power: Pava,0=900 kW

- Required power: Preq,0=780 kW

At altitude, available power scales with density ratio:

Pavah=Pava,0(ρρ0)

Assume at that altitude the density ratio is:

ρρ0=0.82

and required power increases by 6% due to performance degradation:

Preqh=1.06 Preq,0

Excess power:

Pexc=Pava- Preq

What is the new excess power?

a) Pexc≈-6 kW

b) Pexc≈0 kW

c) Pexc≈+32 kW

d) Pexc≈+96 kW