# HLK201 Helicopter Design – Solution Sheet

Questions Set 1: Fundamentals of Helicopter Design

This solution sheet provides the correct option and a short technical justification for each question, including worked calculations where applicable.

## Q1 (7 pts)

Correct answer: c) Main rotor

The main rotor produces the majority of lift by generating aerodynamic force on the rotating blades. The tail rotor is primarily for anti-torque/yaw control.

## Q2 (8 pts)

Correct answer: c) Pedals

Pedals command tail rotor (or anti-torque) thrust, which creates a yawing moment about the vertical axis.

## Q3 (8 pts)

Correct answer: b) 200 N/m²

Disk loading is thrust per rotor disk area.

Calculation / Steps:

Use DL = T / A.

Given T = 10,000 N and A = 50 m².

DL = 10,000 / 50 = 200 N/m².

Common pitfall: Mixing up area units (m²) or using rotor radius instead of rotor disk area.

## Q4 (15 pts)

Correct answer: c) Symmetric

In ideal hover, there is no forward speed, so the inflow and velocity distribution across the rotor disk is (approximately) axisymmetric/symmetric.

## Q5 (8 pts)

Correct answer: c) Hover

Hover is the flight condition where the helicopter maintains position (zero groundspeed) while producing lift equal to weight.

## Q6 (8 pts)

Correct answer: b) Profile power

Profile power is the power required to overcome rotor blade profile drag. It exists whenever the rotor turns, even in hover.

## Q7 (15 pts)

Correct answer: c) Hover requires maximum induced power

Induced power is associated with producing thrust (lifting the aircraft) and is highest in hover because there is no translational lift to reduce induced velocity.

## Q8 (8 pts)

Correct answer: c) 1.225 kg/m³

1.225 kg/m³ is the standard sea-level air density used in many performance calculations (ISA standard).

## Q9 (8 pts)

Correct answer: b) Tandem rotor

Tandem rotors counter-torque by using two main rotors rotating in opposite directions, eliminating the need for a tail rotor.

## Q10 (15 pts)

Correct answer: b) Changes blade angle of attack equally

Collective changes pitch of all main-rotor blades by the same amount, increasing/decreasing total rotor thrust (and required power) without tilting the rotor disk.

Common pitfall: Confusing collective (total thrust control) with cyclic (disk-tilt/pitch-roll control).

## Q11 (20 pts)

Correct answer: b) (per provided key)

Using momentum theory in hover: v_i = sqrt(T/(2 ρ A)) and P_i = T v_i, with A = πR².

Calculation / Steps:

Compute disk area: A = πR² = π·6² = 113.10 m².

Compute induced velocity: v_i = sqrt(40,000 / (2·1.225·113.10)) ≈ sqrt(144.35) ≈ 12.0 m/s.

Compute induced power: P_i = T v_i = 40,000·12.0 ≈ 480 kW.

Common pitfall: The computed (ideal) result (~12.0 m/s, ~480 kW) does not exactly match the listed options in the question. The closest option by v_i is (d). If you want a single correct MCQ option, adjust either R, A, or the answer choices.