# HLK201 Helicopter Design – Solution Sheet

Questions Set 2: Rotorcraft Performance and Control

Correct options are given with short technical justification. Mathematical items include worked steps.

## Q1 (7 pts)

Correct answer: b) Translational lift is absent

In hover there is no forward velocity through the rotor disk. Without translational lift, induced velocity (and induced power) is higher, driving higher power required.

## Q2 (8 pts)

Correct answer: b) Advancing blade

In forward flight, the advancing blade sees a higher relative airspeed (rotational speed plus forward speed), while the retreating blade sees a lower relative airspeed.

## Q3 (15 pts)

Correct answer: b) Hover

Hover ceiling is typically lower than forward-flight ceiling because hover requires higher induced power and lacks translational lift.

## Q4 (8 pts)

Correct answer: c) 90 kW

Induced power in hover is P_i = T v_i.

Calculation / Steps:

Given T = 15,000 N and v_i = 6 m/s.

P_i = 15,000·6 = 90,000 W = 90 kW.

Common pitfall: Forgetting to convert W to kW (divide by 1000).

## Q5 (8 pts)

Correct answer: b) Momentum Theory

Momentum theory models the rotor as an actuator disk imparting momentum to the airflow.

## Q6 (15 pts)

Correct answer: d) Blade twist distribution

Momentum theory provides disk-averaged induced quantities (e.g., induced velocity, thrust) but does not determine detailed blade geometry like twist or chord distribution.

## Q7 (8 pts)

Correct answer: b) Thrust per unit area

Disk loading is defined as DL = T/A.

## Q8 (8 pts)

Correct answer: c) Cyclic

Cyclic changes blade pitch as a function of azimuth, tilting the rotor disk to generate a lateral/longitudinal force component that counters overturning moments in forward flight.

## Q9 (15 pts)

Correct answer: b) Profile power

Profile power is associated with rotor blade profile drag; parasite power is associated with fuselage/airframe drag.

## Q10 (8 pts)

Correct answer: c) To carry higher structural loads

Blade roots experience the highest bending moments and centrifugal loads, so they are structurally reinforced (thicker sections).

## Q11 (20 pts)

Correct answer: a) (per provided key)

Excess power is the difference between available and required power: P_exc = P_ava − P_req.

Calculation / Steps:

Available power at altitude: P_ava(h) = 900·(ρ/ρ0) = 900·0.82 = 738 kW.

Required power at altitude: P_req(h) = 1.06·780 = 826.8 kW.

Excess power: P_exc = 738 − 826.8 = −88.8 kW.

Common pitfall: The computed value (−88.8 kW) does not match the provided answer choices (−6, 0, +32, +96 kW). To make the MCQ consistent, revise the options or the scaling assumptions.