# HLK201 Helicopter Design – Solution Sheet

Questions Set 3: Rotorcraft Design and Optimization

These items focus on scaling laws, trade-offs, and theory limits. Calculations are shown explicitly.

## Q1 (8 pts)

Correct answer: b) 10%

From v_i = sqrt(T/(2ρA)), with ρ and A constant, v_i ∝ sqrt(T).

Calculation / Steps:

If thrust increases by 21%, T2/T1 = 1.21.

v_i,2 / v_i,1 = sqrt(1.21) ≈ 1.10.

So induced velocity increases by about 10%.

Common pitfall: Assuming v_i scales linearly with T (it scales with sqrt(T)).

## Q2 (12 pts)

Correct answer: c) 73%

Given P_i ∝ v_i³, a 20% increase in v_i implies (1.2)³ increase in induced power.

Calculation / Steps:

Compute multiplier: (1.2)³ = 1.728.

Increase = 1.728 − 1 = 0.728 ≈ 73%.

Common pitfall: Using a linear approximation for a cubic scaling law.

## Q3 (8 pts)

Correct answer: c) 0.71

For fixed thrust, P_i ∝ 1/√A. If A doubles, P_i scales by 1/√2.

Calculation / Steps:

A_B = 2 A_A.

P_i,B / P_i,A = 1/√2 ≈ 0.707 ≈ 0.71.

## Q4 (14 pts)

Correct answer: c) Increase induced power

Reducing rotor radius reduces area A, increasing disk loading and induced velocity, which increases induced power in hover.

## Q5 (8 pts)

Correct answer: c) To avoid compressibility effects and noise

High tip Mach numbers introduce compressibility/shock formation, increased noise, vibration, and efficiency loss; hence practical limits are used.

## Q6 (16 pts)

Correct answer: c) Induced power dominates in hover

At zero forward speed, parasite power is minimal; induced power is the dominant term for producing thrust. Profile power is present but typically smaller than induced power in hover.

## Q7 (8 pts)

Correct answer: c) Climb rate, range, endurance

Excess power (P_exc = P_ava − P_req) directly governs climb capability and the margin for mission performance.

## Q8 (18 pts)

Correct answer: c) Momentum Theory alone is sufficient for blade design (FALSE)

Momentum theory gives disk-averaged quantities; blade design requires geometry/airfoil effects (e.g., via Blade Element Theory or BEMT).

## Q9 (8 pts)

Correct answer: c) Reduce induced power

For constant thrust, increasing disk area reduces induced velocity and thus reduces induced power.

## Q10 (20 pts)

Correct answer: c) Larger rotor disk area

Hover efficiency improves when disk loading decreases. Larger rotor disk area (for the same weight/thrust) reduces induced velocity and induced power.

## Q11 (20 pts)

Correct answer: b) 0.77

For constant thrust and density, P_i ∝ 1/√A and A ∝ R², so P_i ∝ 1/R.

Calculation / Steps:

R_B = 1.30 R_A.

P_i,B / P_i,A = R_A / R_B = 1 / 1.30 ≈ 0.769 ≈ 0.77.

Common pitfall: Forgetting that area scales with R², so power scales with 1/R (not 1/R²).